AMERICAN INTERNATIONAL UNIVERSITY-BANGLADESH

Faculty of Engineering

# Lab Report

Experiment # 08

Experiment Title: Implementation of a weather forecast system using the ADC modules of an Arduino.

| Date of Perform: | 21 December 2025 | Date of Submission: | 28 December 2025 |
| --- | --- | --- | --- |
| Course Title: | MICROPROCESSOR AND EMBEDDED SYSTEMS LAB |  |  |
| Course Code: | EEE 4103 | Section: | O |
| Semester: | Fall 2025-26 | Degree Program: | BSc in CSE |
| Course Teacher: | Dr. Md. Rukonuzzaman |  |  |

Declaration and Statement of Authorship:

- I/we hold a copy of this Assignment/Case Study, which can be produced if the original is lost/damaged.

- This Assignment/Case Study is my/our original work and no part of it has been copied from any other student’s work or from any other source except where due acknowledgment is made.

- No part of this Assignment/Case Study has been written for me/us by any other person except where such collaboration has been authorized.

by the concerned teacher and is acknowledged in the assignment.

- I/we have not previously submitted or currently submitting this work for any other course/unit.

- This work may be reproduced, communicated, compared, and archived to detect plagiarism.

- I/we permit a copy of my/our marked work to be retained by the Faculty Member for review by any internal/external examiners.

- I/we understand that Plagiarism is the presentation of the work, idea, or creation of another person as though it is your own. It is a form of cheating and is a very serious academic offense that may lead to expulsion from the University. Plagiarized material can be drawn from, and presented in, written, graphic, and visual forms, including electronic data, and oral presentations. Plagiarism occurs when the origin of the source is not appropriately cited.

- I/we also understand that enabling plagiarism is the act of assisting or allowing another person to plagiarize or copy my/our work.

* Student(s) must complete all details except the faculty use part.

** Please submit all assignments to your course teacher or the office of the concerned teacher.

| Group # 06 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sl No | Name | ID | PROGRAM | SIGNATURE |
| 1 | Chinmoy Guha | 22-48056-2 | BSc CSE |  |
| 2 | Suvra Chakraborty | 22-48067-2 | BSc CSE |  |
| 3 | Mohammad Ansar Uddin | 22-47975-2 | BSc CSE |  |
| 4 | Arfanul Hoque Nehal | 23-54270-3 | BSc CSE |  |
| 5 | Mashkur Ibtesham Wazed | 23-54266-3 | BSc CSE |  |

| Faculty use only |  |  |
| --- | --- | --- |
| FACULTY COMMENTS | Marks Obtained |  |
|  | Total Marks |  |

# Table of Contents

| # Experiment Title | # 3 |
| --- | --- |
| # Objectives | # 3 |
| # Equipment List | # 3 |
| # Circuit Diagram | # 4 |
| # Code/Program | # 5 |
| # Hardware Output Results | # 6 |
| # Experimental Output Results | # 7 |
| # Simulation Output Results | # 8 |
| # Answers to the Questions in the Lab Manual | # 9 |
| # Discussion | # 12 |
| # References | # 13 |

Experiment Title: Implementation of a weather forecast system using the ADC modules of an Arduino.

# Objectives:

The objectives of this experiment are to-

- Familiarize the students with the Micro-controller-based weather forecast system

- Measure environmental parameters, such as temperature, pressure, and humidity

- Simulate a microcontroller-based system using Proteus simulation tool.

# Equipment List:

# 1) Arduino Uno Board

# 2) BMP180/MPL115A

# 3) Inches96 inch OLED 128X64

# 4) Breadboard

# 5) Jumper Wires

# Circuit Diagram:

The diagram here illustrates the wiring setup for a weather prediction device built around a microcontroller. The BMP180 sensor handles readings of temperature and air pressure to help forecast upcoming weather conditions. There's also a 0.96-inch OLED screen with 128x64 resolution that lets you monitor shifts in those values and see the weather outlook. The sensor and display both hook up to the Arduino's VCC for power, and they're grounded together too. For communication, the SDA and SCL pins on each connect to the A4 and A5 pins on the Arduino, in that sequence.

Fig. 1 Hardware circuit diagram for the traffic light

# Code/Program:

Arduino Code for Weather Forecasting System:

#include <SPI.h>

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#include <Adafruit_BMP085.h>

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT);

Adafruit_BMP085 bmp;

#define SEALEVELPRESSURE_HPA (101500)

float simpleweatherdifference, currentpressure, predictedweather, currentaltitude;

void setup() {

// put your setup code here, to run once:

display.begin(SSD1306_SWITCHCAPVCC, 0x3C);

if (!bmp.begin()) {

Serial.println("Could not find a valid BMP085 sensor, check wiring!");

while (1) {}

}

}

void loop() {

// put your main code here, to run repeatedly:

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0,5);

display.print("BMP180");

display.setCursor(0,19);

display.print("T=");

display.print(bmp.readTemperature(),1);

display.println("*C");

/*prints BME180 pressure in Hectopascal Pressure Unit*/

display.setCursor(0,30);

display.print("P=");

display.print(bmp.readPressure()/100.0F,1);

display.println("hPa");

/*prints BME180 altitude in meters*/

display.setCursor(0,40);

display.print("A=");

display.print(bmp.readAltitude(SEALEVELPRESSURE_HPA),1);

display.println("m");

delay(6000);

display.display();

currentpressure=bmp.readPressure()/100.0;

predictedweather=(101.3*exp(((float)(currentaltitude))/(-7900)));

simpleweatherdifference=currentpressure-predictedweather;

//display.clearDisplay();

display.setCursor(0,50);

if (simpleweatherdifference>0.25)

display.print("SUNNY");

if (simpleweatherdifference<=0.25)

display.print("SUNNY/CLOUDY");

if (simpleweatherdifference<-0.25)

display.print("RAINY");

display.display();

delay(2000);

}

Explanation:

This script runs on an Arduino board and employs a BMP085 module to capture readings for temperature, atmospheric pressure, and elevation. These values are shown on a compact OLED panel via the Adafruit GFX and SSD1306 packages. In the setup routine, it gets the OLED screen and BMP085 up and running; if the module isn't detected, it shows an alert. The main loop, which cycles indefinitely while the device has power, kicks off by wiping the screen clean and adjusting font size and hue. It proceeds to output the temp, pressure, and height data pulled from the BMP180 onto the display. Following a 6-second pause, it refreshes the view. Then, it grabs the latest pressure value from the sensor and figures out a weather forecast tied to the present height. The gap between actual pressure and this forecast gets saved in a var called "simpleweatherdifference." Depending on that figure, it outputs "SUNNY," "SUNNY/CLOUDY," or "RAINY" on the screen. Lastly, it hangs for 2 seconds before circling back to redo the whole sequence.

Hardware Output Results:

Fig: Experimental setup for a Weather Forecasting using BMP180 and OLED

Experimental Output Results:

Fig. Output showcasing the code works.

Simulation Output Results:

Fig: Temperature = 25.3℃, Pressure = 1013.2hPa, Altitude = 15.1m, Weather = SUNNY

Fig: Temperature = 29.3℃, Pressure = 1009.96hPa, Altitude = 42.1m, Weather = SUNNY

Fig: Temperature = 35.7℃, Pressure = 1006.4hPa, Altitude = 72.0m, Weather = SUNNY

Simulation Methodology:
For the weather forecasting system simulation, an Arduino Uno board was placed on the Proteus workspace along with a BMP180 sensor for measuring temperature and barometric pressure, and a 0.96-inch 128x64 OLED display for showing parameter changes and predicted weather. Both the sensor and OLED were connected to the VCC pin of the Arduino for power supply and to the GND pin for grounding. The serial data (SDA) pins of the sensor and display were linked to the A4 pin of the Arduino, while the serial clock (SCL) pins were connected to the A5 pin. Upon running the simulation with the loaded code, the OLED displayed real-time readings such as temperature at 35.7°C, pressure at 1006.4 hPa, altitude at 72.0 m, and a "SUNNY" forecast, confirming the system's ability to process sensor data and output basic weather predictions.

Next, to test variations in environmental conditions, the simulation parameters were adjusted to simulate different atmospheric inputs to the BMP180 sensor. In one scenario, the display updated to show temperature at 25.3°C, pressure at 1013.2 hPa, altitude at 15.1 m, and maintained a "SUNNY" prediction based on the pressure difference calculation. During another run, the readings shifted to temperature at 29.3°C, pressure at 1019.0 hPa, altitude at 42.1 m, again resulting in a "SUNNY" output, demonstrating the code's logic for forecasting under stable pressure conditions while the display refreshed after a programmed delay.

# Discussion:

The main objectives of this project were to familiarize participants with a microcontroller-driven weather prediction setup and build their understanding of its operations. Through the activity, we learned about key atmospheric factors like temperature, pressure, and humidity by directly assessing them. It also offered practical skills in building and deploying a setup capable of collecting and processing information on climatic trends. To start, software was developed for the device. This program powers a monitoring station that relies on the Adafruit BMP180 module to detect temperature, pressure levels, and height. Outputs appear on a 128x64 OLED panel via the Adafruit_SSD1306 framework. The BMP180 links to the Arduino through the I2C interface. As an absolute pressure detector, the BMP180 gauges barometric readings to infer potential weather shifts. Accurate forecasting needs a fixed spot and about 2-3 hours for thorough trend evaluation. Place the unit in a shielded spot away from heavy winds or thermal disturbances. The BMP180 handles broad temperature variations from 0 to 85°C without needing recalibration. We carried out the test as planned, capturing various sample readings. The system tracks temperature, derives altitude from data, estimates humidity, and projects if conditions will be clear, overcast, or stormy. We ran simulations to cross-check the physical outcomes, and both sets aligned closely. Overall, it's reasonable to say the endeavor succeeded.

# References:

1.Baskoro, Farid & Rohman, Miftahur & Nurdiansyah, Aristyawan. (2020). Serial Peripheral Interface (SPI) Communication Application as Output Pin Expansion in Arduino Uno. INAJEEE Indonesian Journal of Electrical and Eletronics Engineering. 3. 63. 10.26740/inajeee. v3n2.p63-69. 3)

2.O. Loskutov, A. B. Karakozov and P. V. Nekrasov, "Automated test setup for functional and parametrical control of microcontrollers with internal ADC," 2016 International Siberian Conference on Control and Communications (SIBCON),  Moscow, Russia, 2016,  pp.  1-4,  doi: 10.1109/SIBCON.2016.7491799.